** DATA STRUCTURE DOCUMENTATION **

THIS DOC. DESCRIVES THE STRUCTURE OF THE DATA

USED IN AX68K.COM

1. BUFFER MEMORY STRUCTURE

1-1. SOURCE BUFFER

STRUCTURE IS AS FOLLOWS

buffer top	-----------------------

sources

buffer start	------------------------

sources

buffer limit	------------------------

sources

buffer end	------------------------

Initially, source are read from disk,

and transfered to the area (from buffer start to end ).

If the source buffer pointer exceed buffer limit address,

then transfer the block (from buffer limit to end)

to the area (from top to start).

After the transfer, the buffer pointer must be compensated

so as to indicate the correct address.

Therefore the area size (end-limit) must be equal to (start-top).

Then, the new next source are read from disk

to the area (from start to end).

1-2  PRN or OBJ BUFFER

top		----------------------

data

middle		----------------------

data

limit		----------------------

data

end		----------------------

Like as the source buffer the structure is circular.

But the starting point is different.

Initially, AX68K store the data from the top of the buffer.

If exceed limit then write the block (from top to limit)

to disk.

And transfer (limit to end) to (top to middle).

Then compensates pointer.

2. LABEL DATA STRUCTURE

2-1.  LABEL BUFFER

The followings are label buffer structure.

top	---------------------

sequential area

bottom	---------------------

contiguous area

end	---------------------

2-2.  HASH function

Initially, the label word is allocated to the sequential area

by a hash value.

The hash function is the lower 5 bits of

the summation of the label word characters (ASCII).

Therefore the value is 0~31.

As a metter of cource, the conflict of the same hash address will occur.

But in the point of view that the complicated hash function

costs more calculation time ,and that the destribution of

the label word character can not be estimated,

I adopted this function.

2-3.  Sequential buffer area

The label with same hash are stored to the same buffer

in order.

The proper delimiter is inserted to distinguish

the label word and 32 bit data.

And a proper terminater is positioned to indicate the data end.

The methode to find the gap between the former label and the next one

is to search the delimiter and skip 4 byte (32 bit data).

ex.

DATA FORMAT

ABRREVIATIONS

<LAB>=LABEL WORD CHARACTER PART

<DEL3>=0, DELLIMITER BETWEEN CHAR.PART

AND DATA PART

<DATA>=LABEL VALUE

<TERM2>=0FFH , END TERMINATER

<TERM3>=0FEH , INDEX INDICATE TERMINATER

<ID>=INDEX FIELD (2 BYTE)

FORMAT

EXAMPLE 1

ONLY SINGLE LABEL

<LAB><DEL3><DATA><TERM2>

EX. 2

2 LABELS

<LAB><DEL3><DATA><LAB><DEL3><DATA><TERM2>

According to the hash value, the sequential area

is partitioned to 32 blocks.

As mentioned above, the labels are stored to

from the entry of the block, in the order of resistration.

i.e. if the label word is 'ABC' then the hash=('A'+'B'+'C') AND 1FH

=(41H+42H+43H) AND 1FH =06H

The store block=block 6 .

(the sequential area consists of block0,block1,....,block31)

If any label already have stored in the block 6 ,

for example

entry of block 6	<LAB><DEL3><DATA><LAB><DEL3><DATA><TERM2>

The next label 'ABC' and it's value (for example =023458E7H)

will be resistered as follows.

entry of block 6	<LAB><DEL3><DATA>

<LAB><DEL3><DATA><'ABC'><DEL3=0><023458E7H><TERM2=0FFH>

2-4.  Contiguous buffer area

If once the sequential block had been filled,

the next label to be stored is registered

to the contiguous area with the following format.

EX. 3

AMOUNT OF LABELS

<LAB><DEL3><DATA>......<LAB><DEL3><DATA><TERM3><ID><LAB><DEL3>...<TERM2>

In the EX. 3 ,the data up to <ID> are contained in a sequential block.

But the latter data is in the contiguous buffer.

So the correct description is as follows.

Label buffer top	----------------------

(sequential buffer top)

<The first block (=block 0) area>

End of the first block	----------------------

Top of the block 1	----------------------

.....................

.....................

.....................

Entry of a block	--------------------

<LAB><DEL3><DATA><LAB><DEL3><DATA>.......

<LAB><DEL3><DATA><TERM3><ID>

<empty area>

this small area is empty because it has not enough size

to store next label data.

End of the block	----------------------

Top of the next block	----------------------

.....................

.....................

.....................

End of the last block (=block 31)-------------

Top of the contiguous area -------------------

<LAB><DEL3><DATA><TERM3><ID><LAB><DEL3><DATA><TERM3><ID>.....

....<LAB><DEL3><DATA><TERM3><ID><TERM2>

<large empty area for storage>

End of the label buffer -----------------------------

In the contiguous area, many label data with different hash

are stored mixedly.

Any label data is linked from other label,

and it links to another label or <TERM2> (=end mark ).

Consequently, the last label of a sequential block

links a label somewhere in the contiguous area.

The address of the linked label is indicated

by the 2 bytes long link index field, <ID>.

The linked chain of different hash does not join nor cross

except for the END terminater.

i.e.

EX. 4

LAST INDEX POINTS TERM2 ADDR.

.....<LAB><DEL3><DATA><TERM3><ID><TERM2>

<ID> OF LABELS WHICH HAVE OTHER HASH, POINT SAME TERM2

Therefore all of the last <ID> of each hash chain

point the same address of <TERM2>.

3. Hash index table

This table is not always required.

(But for hole contiguous buffer structure, it is needed

to indicate the hash block entry.)

In AX68K, this table is used because of the

fast label registration.

3-1.  Table structure

Table top	-------------------------

<LINK FLAG><INDEX DATA>

<LINK FLAG><INDEX DATA>

...................

...................

<LINK FLAG><INDEX DATA>

Table end	-------------------------

The table consists of 32 <LINK FLAG><INDEX DATA>.

3-2.  Data structure

1 byte non zero <LINK FLAG> indicates

that the hash chain is already linked.

i.e. if the hash chain is sequential type yet,

then <LINK FLAG>=0.

If linked, the 2 bytes <INDEX DATA> represents

the most recent indexed address.

As previously descrived, this table is applied

merely to point out the last tail of the chain,

to accelerate the regitration of the new label.

4. OP-MNEMONIC table

Please refer DATA.LIB too.

This table contains OP-CODE type and OP-MACINE CODE of each mnemonic.

4-1.  data format

<MNEMONIC><DEL3><TYPE><MACINE CODE or DATA><SUB DATA>

4-2.  table structure

The data is ordered of the hash number (0~31).

MNM0:	DB 'mnemonic',0,type,op-code,sub-code

..............

..............

DB 0FFH	;hash=0 block end mark

MNM1:	DB ..........

.............

DB 0FFH	;block end

MNM2:	.............

......................

......................

MNM1F:	DB ...........

..............

DB 0FFH

;table end

If you want to register the new MNEMONIC

, you must store it in the correct hash block.

This procedure is automatically done by the HASH.COM .

The HASH.COM change the DATA2.LIB .

Please type HASH and cr.

(Caution ; HASH.COM rewrite the DATA2.LIB, so please

copy the DATA2.LIB before the rewrite.)

The OP-TYPE number is determined by the operation addressing.

That is , the operations having the same addressing belong

to the same type.

ex.

ADD and SUB belong to the same type.

But ADD and AND do not.

Because, AND <ea>,Dn permits <ea> only data addressing modes.

On the other hand ADD <ea>,Dn allows all addressing modes.

See MC68000 manual.

#############################################################################################################################