TurboDOS drivers for the IBM Personal Computer

The purpose of this document is to give a brief explanation and

description of the TurboDOS drivers for the IBM Personal Computer as

installed on the enclosed disks.  The actual drivers are provided in

source form and OEM's are envited to use them freely.  Software 2000

would  greatly  appreciate  enhancement  suggestions,  but  will not

assume  any  responsibility  for  long  term  maintenance  of  these

drivers.

Since there are many PC look-alikes claiming at least ROM level

compatability, it was decided to make as  few assumptions  about the

hardware as possible.  All peripherals  interesting to  TurboDOS are

supported in the PC's ROM,  so TurboDOS  drivers rely  completely on

the ROM drivers for hardware dependent operations.

The ROM supports one standard disk format of 512  byte sectors,

8 per side, one or two sides, double density.  A TurboDOS format was

chosen to allow easy access to  PC-DOS format  disks in  addition to

providing  bootup compatability  with the  ROM.  Unfortunately, this

format is not the most efficient and it's use  should be  limited to

bootup and certain media conversion tasks.

Bootup is accomplished on the PC  by the  ROM reading  track 0,

sector  1  from  drive 'A'  into memory  address 0000:7C00  and then

transferring control there.  Since the  ROM can  read only  one disk

format, the boot driver assumes the same restriction in an effort to

keep  it's  size  to  a  minimum.  The  resulting MPBIPC  and OSBOOT

together are small enough to reside in the one reserved sector area.

The memory specification table, MSTIPC, defines a  256K machine

with TurboDOS to reside at 0050:0000.  The  ROM makes  use of  a 256

byte area  at 0040:0000,  so these  locations must  be avoided.  For

other memory configurations, this table will have to be changed.

Real  time  clock,  console,  and  disk  driver  initialization

routines  are  called  from  the  HDWNIT  entrypoint  in NITIPC.  As

additional drivers are added,  their initialization  entrypoints may

also be called from this module.

The ROM  allows a  user routine  to be  envoked each  real time

clock tick.  This facility is utilized in  RTCIPC to  provide system

ticks, one second markers, and ISRXIT support.  This module is coded

specifically for the IBM PC ROM and may need modification if used on

other  machines.  If  in  doubt,  start  out  with  RTCNUL  which is

hardware independent.

The console  driver, CONIPC,  uses a  poll routine  to retrieve

characters from the ROM  keyboard driver.  Type-ahead  buffering and

attention character monitoring are incorporated.  Console  output is

handled completely by the ROM video driver in TTY mode.

The floppy disk driver accomodates the 1024 byte sector, 48 TPI

TurboDOS disk formats in addition to the PC compatable formats.  The

IBM PC ROM supports variable disk formats  via a  pointer to  a disk

parameter table.  DSKIPC manipulates this pointer to  various tables

which  are  extensions  to  the  usual  disk  specification  tables.

Diskette formatting is supported by FMTIPC which is  linked together

with FORMAT and FMTNEC.

Two  list  drivers  are  supplied,  LSTPPA  and LSTACA.  LSTPPA

assumes a parallel printer attached to the parallel printer adapter.

LSTACA  assumes  a  serial  printer  attached  to   an  asynchronous

communications adapter.

No attempt has been made to support any of the IBM XT features,

but it was verified that the drivers supplied would boot up  and run

successfully from the floppy drive on an XT.

###################################################################################