(The following documentation contains design objectives which

were written before work was begun.  It also contains detailed

descriptions of completed functions.)

FUNCTIONAL DESCRIPTION

Rather than one very powerful CPU controlling everything, several

smaller CPU's will be integrated into a network.  This allows the

individual CPU's to be more closely matched to their particular

functions.  It also allows construction to proceed in a modular

fashion.

MAIN CPU

This CPU is responsible for determining what actions to take.  It

must be constantly aware of exact vehicle position in relation to

surroundings, including temporary obstacles.  It accepts and

interprets encoded speech commands from the speech processor CPU

and sends encoded speech responses to the speech processor for

utterance.  The main CPU issues motion commands to the base CPU

and requests reports on motion progress and vehicle status.  It

also accepts asynchronous notification of emergency events.

Initially, the main CPU will be located off-board.  It will

communicate with the vehicle via a serial link, either radio or

umbilical cord.

BASE CPU

The base CPU is responsible for all hardware on the vehicle base,

except the speech components.  This CPU controls the stepper

motors and DC drive motors and monitors their position.  It

monitors all bumper and limit switches.  When a switch closes,

the corresponding motor is stopped first, then the main CPU is

notified of the aborted command.  The battery charge level is

monitored, and charging operations proceed under its control.

Distance-to-object and direction-of-sound are calculated by the

base CPU.  The radio link to the main CPU, and the LED link to

the platform CPU are maintained by the base CPU.  A parallel port

connects it to the speech CPU.  Base CPU memory is used for

message buffering.

SPEECH CPU

The speech CPU interprets sounds and encodes them into word-

codes.  These word-codes are passed to the main CPU (through

thebase CPU's radio link) for interpretation of actual meaning.

The speech CPU receives word-codes from the main CPU, which it

utters via the speech synthesizer.

PLATFORM  CPU

The platform CPU is responsible for all components mounted on the

rotating platform, except the video digitizer.  It communicates

with the base CPU via LED's mounted in the support shaft.

Rotation of the platform is performed by the base control CPU,

upon request of the platform CPU.

VIDEO CPU

The video CPU captures digitized images, evaluates them, and

encodes the information it can discern.  It makes lens movement

and focus requests to the platform CPU.  It accepts image-related

requests from the main CPU via the base and platform CPU's.

DETAILED ORGANIZATION

BASE and PLATFORM CPU's

HARDWARE DESCRIPTION

The same design is used for each of these CPU's, since they

perform very similar functions.  This CPU uses a 4MHz Z80

processor with 16K or 64K of memory.  The memory is divided

between RAM and ROM (EPROM).  A Z80 SIO or DART provides two

serial channels.  Two Z80 CTC's handle timing tasks.  Three Z80

PIO's are used for limit switch input and general bit oriented

control functions.  One of the PIO ports is a high-speed bi-

directional parallel link to the speech or video CPU.  Eight

"dumb" output ports are used for stepper motor control, allowing

up to 16 four-phase steppers to be attached.

The processor board contains all the components that must use the

data and address busses, as well as address-decode and clock

generation chips.

Two stepper motor driver boards may be connected to one CPU

board.  Each stepper motor board controls eight four-phase

motors.  All timing and sequencing is handled by the CPU board.

The stepper board only provides output drive amplification.

The support boards contain miscellaneous circuitry specific to

the base or platform hardware.  Both support boards house the

reset circuit and auto-reset one-shot, limit switch buffers, and

LED strobe conditioners.  In addition, the base support board

contains FSK timing and radio link support, including transmit

solenoid and walkie-talkie board.

The power board attached to the base CPU contains the DC motor

drivers and direction control, battery monitor and charge

circuitry, and voltage regulators.  The +12VDC from the battery

is regulated to +5VDC by a swithced-mode regulator.  A small

linear regulator provides +9VDC for the walkie-talkie.

AUTO-RESET

A primary design requirement states that the system will stop all

motion in the event of an unexpected error.  In this case,

"unexpected" means any error that the CPU does not monitor or for

which it does not have a recovery routine.  This will prevent

damage to the vehicle and to its environment, as well as limit

unnecessary battery drain.  To accomplish this requirement, an

"auto-reset" mechanism is employed.  A 74LS123 retriggerable

monostable is gated to the system-reset circuit.  Its output

pulse is initiated by the system-reset line.  It is kept active

by retriggering the one-shot via a parallel port bit.  If the

pulse ever times out, the system will perform a hardware reset.

If a software or hardware error prevents the program from

toggling the trigger bit within the allotted time, the resulting

reset will stop any motor activity and cause system diagnostics

to be performed.  The one-shot design was chosen because it runs

independently of the CPU.  A counter circuit running from the

system clock, for example, would fail to function if the CPU quit

because the system clock failed.

################