PBUFF/ZLOAD EPROM MEMORY MAP

TO SELECT

������������������������������������������������������������������������ͻ

�3FFFĿ                          �             �        �                �

�     �                          �             �        �                �

�     �                 (BANK 3) �             �        �                �

�     �� � � � � � � � � � � � � �             � PBUFF  �                �

�     �8K                        �J6 LOW       �        �                �

�     �PBUFF5P1E.ROM    (BANK 2) �             �        �                �

�2000��                          �PIN 26 HIGH  �        �  27128 EPROM   �

�������������������������������������������������������Ĺ      16K       �

�1FFFĿ                          �J6 HIGH      �        �   PBUFF.ROM    �

�     �4K    RUN POSITION        �             �        �                �

�1000��ZLOAD JUMP.ROM   (BANK 1) �PIN 26 LOW   �        �                �

����������������������������������������������Ķ ZLOAD  �                �

�0FFFĿ4K    LOAD POSITION       �J6 LOW       �        �                �

�     �ZLOAD LOADER.ROM (BANK 0) �             �        �                �

�0000��                          �PIN 26 LOW   �        �                �

������������������������������������������������������������������������ͼ

ZLOAD VERSION 2.0 MEMORY MAP AND JUMP ROM DETAILS.

FFFFĿ

� DRAM MEMORY. NOT USED FOR ZLOAD

C000��

BFFFĿ

� MIRROR IMAGE OF 8000-9FFF

A000��

����������������������������������������������������������������ͻ

�9FFF���TOP OF MEMORY�Ŀ(MEMORY TEST BYTE) �                     �

�9FF0�����������������Ĵ128 BYTES          �                     �

�     � ZLOAD STACK ETC�RESERVED FOR ZLOAD.�                     �

�9F80�������������������������������������Ķ 6264 STATIC RAM     �

�9F7FĿ                                    �                     �

�     �8064 BYTES USEABLE.                 �                     �

�8000��LOAD ADDRESS.                       �                     �

����������������������������������������������������������������ͼ

7FFFĿ

�EPROM MIRROR IMAGE

6000��

5FFFĿ

�EPROM MIRROR IMAGE

4000��

3FFFĿ

�EPROM MIRROR IMAGE

2000��

����������������������������������������������������������������ͻ

�1FFFĿ                                    �                     �

�     �    4K    RUN POSITION              �                     �

�1000��    ZLOAD JUMP.ROM   (BANK 1)       �      2764 EPROM     �

������������������������������������������͹                     �

�0FFFĿ    4K    LOAD POSITION             �                     �

�     �    ZLOAD LOADER.ROM (BANK 0)       �       ZLOAD.ROM     �

�0000��                                    �                     �

����������������������������������������������������������������ͼ

JUMP.ROM (4K)

ADDRESS 1000H BECOMES 0000H WHEN RUN POSITION SELECTED...

ADDRESS   INSTRUCTION      COMMENTS

1000H     JUMP TO 8000H    JUMP TO START CODE AT RAM 8000H

ALSO RST 0H (POWER UP & RESET)

1008H     JUMP TO 8008H    RST 8H

1010H     JUMP TO 8010H    RST 10H

1018H     JUMP TO 8018H    RST 18H

1020H     JUMP TO 8020H    RST 20H

1028H     JUMP TO 8028H    RST 28H

1030H     JUMP TO 8030H    RST 30H

1038H     JUMP TO 8038H    PIN 16 INT (555 TIMER ON PBUFF)

ALSO RST 38H

1066H     JUMP TO 8066H    PIN 17 NMI (TEST ON PBUFF)

1080H     ASCII TEXT       ID ONLY FOR THIS BANK

'JUMP.ROM VERSION 1.0'

1FFFH     LAST BYTE