LET'S INCREASE OUR EPROM MEMORY   UPDATED 14-Apr-94 by Don McKENZIE.

NOTE ***** DON'T CONFUSE THE DUAL PBUFF_ZLOAD 27128 OR LARGER TYPE EPROMS

OF VERSION 2.x ZLOAD WITH THE MEANS OF INCREASING EPROM SIZE AS GIVEN IN

THIS DOCUMENT. PIN26 OF THIS DUAL EPROM ISN'T USED TO CONNECT TO A13 OR ANY

OTHER ADRESS LINE, IT IS USED TO SELECT EITHER HIGH OR LOW BANK BY

CONNECTING IT TO +5VOLTS OR GROUND.

IN OTHER WORDS, DON'T READ THIS FILE UNTIL YOU TEST YOUR PBUFF OR ZLOAD

BOARD FULLY.

Now consider that a 2764 EPROM only is configured in the board and pin 26 of

the EPROM is connected as normal, that is to +5Volts.

READ ON:

Install a 27128 and simply use A14 as A13. See tail end of this article.

Or add both A14 & A13 to the EPROM if you are not using the A13 output

as a printer out strobe. This allows 27128 or 27256 EPROMs to be used.

Details also given at the end of this article.

Or install a 74LS123 into socket Ey. This method will free the A13

address line completely and preserve the O/P strobe. See bottom of this

article.

_______________________________________________________________

December 30, 1989

Retain Printer output strobe: (An older method.)

To release A13 of the PBUFF CPU for further EPROM expansion is a very

easy job. The output printer strobe only needs to be removed from A13 so

that A13 can be used for more onboard memory decoding, and the strobe

can be attached elsewhere.

RELEASE A13 HARDWARE:-

Connect an 1N914 type small signal diode between pins 17 (NMI) and 18 (HALT)

of the Z80. The bar end connects to the HALT pin. Cut the track between

E13/6 (74LS04) and the output strobe header pin J2/1. Connect J2/1 to the

HALT pin of the CPU. That's simple enough isn't it.

Instead of toggling A13 to issue an output strobe, the HALT pin is used. All

that is required from software to do this is the HALT instruction. The only

problem with this is that the CPU is now in the HALT mode and can only be

restarted with an interrupt. The diode is used to assert the HALT low to the

NMI pin to interrupt the CPU. The diode also isolates the TEST button from

the HALT pin and output strobe.

RELEASE A13 SOFTWARE:-

The NMI interrupt service routine software must be able to tell the

difference between a HALT instruction and a TEST function. A RAM byte

(HLTFLG) is used for this. It must be set to zero on power up for the TEST

function, and set to one for the HALT or output strobe pulse and cleared

after use.

THIS BYTE MUST BE EQUATED AT THE START OF YOUR SOURCE CODE:-

****************************************

* DUBMEM	EQU	CATCH1+1	;November 8, 1989

* HLTFLG	EQU	DUBMEM+1	;December 30, 1989

****************************************

BELOW IS THE CODE FOR THE NMI INTERRUPT SERVICE ROUTINE, FLAG CHECKING, AND

THE RESETTING OF THE HALT FLAG

;PIN 16 555 TIMER (INT)

DEFS	38H-$

INTVEC:	JP	REFRES

****************************************

* HLTSTB:	XOR	A               ;December 30, 1989

*	        LD      (HLTFLG),A      ;CLEAR HALT FLAG

*	        RETN			;RET TO TOGSTB CALLER

****************************************

;PIN 17 TEST (NMI)

DEFS	66H-$

NMIVEC:

***************************************

*       	LD	A,(HLTFLG)      ;December 30, 1989

*	        OR	A               ;TEST HALT FLAG

*        	JP	NZ,HLTSTB       ;IT'S A STROBE

***************************************

LD	BC,0FFFH        ;CONTINUE WITH TEST FUNCTION

CALL	DELAY.......

THE FOLLOWING CODE IS ADDED TO THE TOGSTB ROUTINE.

TOGSTB:	LD	A,(MPORTS)	;8-OCT-87

OR	A		;TEST FOR MULTIPORT..

JR	NZ,PORTST

***************************************

*       	LD	A,1             ;December 30, 1989

*	        LD	(HLTFLG),A      ;SET HALT FLAG

*               DI

*	        HALT                    ;HALT, ISSUE STROBE, NMI CPU

*               EI                      ;DONE ITS THING AND RETURNED

*	        RET

***************************************

;       	LD      B,20H		;SET A13

;	        IN      A,(C)		;STROBE OUTPUT

;	        RET

PORTST:	LD	A,C......

***********************************************

ONCE YOU HAVE RELEASED A13, AND TESTED THE UNIT, THE FOLLOWING MODIFICATIONS

CAN BE DONE TO PBUFF AND ZLOAD.

***********************************************

PBUFF ALTERATIONS....

EPROM PINOUT DIFFERENCES READ MODE....

2764     8K  0000H TO 1FFFH      PIN 1 +5V      PIN 26 N/C     PIN 27 +5V

27128   16K  0000H TO 3FFFH      PIN 1 +5V      PIN 26 A13     PIN 27 +5V

27256   32K  0000H TO 7FFFH      PIN 1 +5V      PIN 26 A13     PIN 27 A14

27512   64K  0000H TO FFFFH      PIN 1 A15      PIN 26 A13     PIN 27 A14

PIN 26 IS CONNECTED TO +5V ON THE PBUFF BOARD. THE 27512 CAN'T BE USED ON

PBUFF WITHOUT FURTHER HARDWARE ADDRESS DECODING LOGIC, BUT THE OTHERS UP TO

32K WILL WORK WITH ONLY THE ABOVE TRACK MODIFICATIONS ONCE YOU HAVE RELEASED

A13 WITH THE CHANGES DETAILED IN THIS ARTICLE..

***********************************************

ZLOAD ALTERATIONS TO SUIT PBUFF.... (Now only applicable to V1.0 ZLOAD)

IF YOU WISH TO INSTALL A 62256 32K STATIC RAM INTO ZLOAD TO MATCH THE SIZE

OF YOUR INCREASED PBUFF EPROM SIZE THEN SOME MINOR ALTERATIONS ARE

REQUIRED:-

PIN 26 OF THE SRAM IS A13, SAME AS THE EPROMS. IF YOU HAVE CONNECTED A13 TO

PIN 26 ON THE PBUFF BOARD THEN THE ARTWORK DOESN'T NEED TO BE ALTERED ON THE

ZLOAD BOARD AS I HAVE ALLOWED FOR THIS CHANGE. A14 USES A DIFFERENT PIN AND

REQUIRES A LITTLE WORK. IF YOU ARE USING A 27128 EPROM THEN IT DOESN'T

MATTER AS THE 62256 WILL BE MAPPED FOR 16K ONLY, BUT FOR 32K, PIN 1 OF THE

SRAM SOCKET IS A14. THIS PIN NEEDS TO ISOLATED AND JUMPERED TO A14 OF THE

HEADER PIN 27. PIN 1 OF THE SRAM SOCKET HAS A +5V RAIL GOING THROUGH IT, SO

TWO TRACK CUTS AND ONE JUMPER ARE REQUIRED TO ISOLATE THIS TRACK.

THE ZLOAD LOADER EPROM ALWAYS REMAINS A 2764 REGARDLESS OF YOUR PBUFF EPROM

SIZE, HOWEVER TO SUPPORT THE 2764 IN THE ZLOAD BOARD FOR THE PBUFF 27256,

REQUIRES ONE CHANGE. PIN 27 OF THE 2764 MUST BE +5V. THIS WILL NEED TO BE

ISOLATED AND JUMPERED.

***********************************************

PBUFF SRAM....

TO INCREASE THE PBUFF SRAM ABOVE 8K REQUIRES ADDITIONAL ADDRESS DECODING

LOGIC. I HAVE USED TO ADVANTAGE PIN 26 BEING A HIGH CHIP SELECT ON THE 6264.

THE 62256 DOESN'T HAVE A HIGH AND A LOW CHIP SELECT.

***********************************************

23-July-92

Use A14 as A13...  This one was pointed out to me by Dr. Paul Webster of

Port Macquarie. N.S.W.

Disconnect the +5V signal from pin 26 of the PBUFF EPROM. This is at

E11/26. The track cut can be done on the solder side of the board.

Connect A14 to E11/26. A14 can be found at the Static Ram socket E14/20.

You will now be able to install a 27128 16K EPROM into the E11 socket.

Only one small problem and it is easily overcome with assembly. The

first 8K of code must be assembled for 0000 to 1FFFH, and the next 8K at

4000 to 5FFFH.

***********************************************

1 August-92

27128 (16K) Installation.

Connect A13 as A13 of the EPROM to allow standard installation of a

27128 EPROM. Inverted A13 Printer output strobe will be no longer

useable.

Disconnect the +5V signal from pin 26 of the PBUFF EPROM. This is at

E11/26. The track cut can be done on the solder side of the board.

Connect A13 to E11/26. A13 can be found at the 74LS04 Inverter E13/5.

You will now be able to install a 27128 16K EPROM into the E11 socket.

The 27128 will now be mapped into the first 16K address range from 0000

to 3FFFH.

***********************************************

1 August-92

27256 (32K) Installation.

Connect A14 as A14 of the EPROM to allow standard installation of a

27256 EPROM. Inverted A13 Printer output strobe will be no longer

useable.

Do the above A13 modification for a standard 27128 EPROM Installation.

Disconnect the +5V signal from pin 27 of the PBUFF EPROM. This is at

E11/27. The track cut can be done on the solder side of the board.

Connect A14 to E11/26. A14 can be found at the Static Ram socket E14/20.

You will now be able to install a 27256 32K EPROM into the E11 socket.

The 27256 will now be mapped into the first 32K address range from 0000

to 7FFFH.

***********************************************

This means that with 2 track cuts and two jumpers, you can increase your

EPROM memory size by four times the original size.

_______________________________________________________________

Install a 74LS123 into socket Ey as follows:

Connect +5V to pins 2 and 3. Connect a 10K resistor between pins 16 and

15. Connect a .01uf capacitor between pins 15 and 14. Connect pin 4 to

J2/1. Connect the output buffer full signal of the 8255 (pin 15) to pin

1 of the 74LS123. Isolate the signal that connect from E13/6 to J2/1.

This is drawn for you below..

_______________________________________________________________________________

New Printer Output Strobe using a 74LS123 in the Ey 16 pin socket position.

NOTE *** +5V and Ground are connected to pins 16 and 8 of socket.

This will release A13 for more EPROM or SRAM.

+5V �������������������������Ŀ

�       �     �     ��

�       �16   �3    � �10K

�     ���������Ŀ   ���

�     �   Ey    �    �

�����Ĵ2      15����Ĵ

�         �  ����� .01uf

� 74LS123 �  �����

OBF               �       14������                  J2/1

8255/15 ���������Ĵ1       4����������������������� Printer O/P Strobe

�         �               �

�����������               �

�8                   �

___                  ���                   �

(A13)                 GND                   �

E13/6   �����������������������X�������������

#

Isolate Track

_______________________________________________________________________________